|  | Document Number | EDCS-15612474 |
| --- | --- | --- |
|  | Based on Template | EDCS-189229 Rev 23 |
|  | Created By | Niels Jensen Jeffery Loo |

Fugazi
Hardware Functional Specification

This is the hardware functional specification for the Fugazi platform.  Fugazi is a Low- End ASR1K platform using a software dataplane.

Reviewers

| # Department | # Name / Title |
| --- | --- |
| SRGBU HW | Kevin Ye / Hardware Manager |

Modification History

| # Revision | # Date | # Originator | # Comments |
| --- | --- | --- | --- |
| 1 | 12/6/2018 | Niels Jensen | Initial checkin |
| 2 | 3/5/2019 | Niels Jensen | Updates for revision 1 |
| 3 | 12/18/2019 | Niels Jensen | Cleanup and remove incorrect and out-of-date stuff. |
| 4 | 4/23/2020 | Niels Jensen | Adding USB description. Minor clarifications added. |
| 5 |  |  |  |

Table of Contents

1	Overview	7

1.1	Risks	7

1.2	Design Considerations	8

1.2.1	NEBS requirement	8

1.2.2	Hardware Process Requirements	8

2	High-Level Description	9

2.1	Configurations	9

2.1.1	Fugazi Bezel Side	9

2.1.2	Fugazi Business (I/O Panel) Side	9

2.2	Performance	10

2.3	Functional Blocks	10

2.4	Theory of Operation	10

2.5	Initiative, Legal, & Regulatory	10

3	Detailed Description	11

3.1	System Block Diagram	11

3.2	Intel Skylake-D Control Plane CPU	11

3.2.1	Control Plane CPU Part Numbers	11

3.2.2	Skylake-D CPU	12

3.2.3	Skylake-D DDR4	12

3.2.4	Skylake-D Flexible HSIO	13

3.2.5	CPU USB Mapping	14

3.2.6	CPU PCIe Mapping	14

3.2.7	CPU PCIe Output Clock Mapping	14

3.2.8	CPU Ethernet Mapping	15

3.2.9	CPU RTC	15

3.3	DDR4 Memory DIMMs	15

3.4	Bootflash	16

3.5	UARTs (Serial Ports)	16

3.5.1	CON RS-232 Transceiver	16

3.5.2	RJ-45 Connector Pin Out	16

3.5.3	USB Console	17

3.6	Front USB Ports	17

3.6.1	USB 3.0 Type-A	18

3.6.2	USB 3.0 Type-C	18

3.6.3	USB High Speed Re-driver	19

3.7	Front Panel Ethernet Ports	19

3.7.1	i211 GE MAC/PHY Management Port	19

3.7.2	Dual 10G MAC, BCM57412	19

3.7.3	Quad GE PHY BCM54194	20

3.7.4	Dual 10G PHY BCM82757	21

3.7.5	1G and 10G PHY SMI Address Map	22

3.7.6	SFP/SFP+ Interface	23

3.8	I2C	24

3.9	Clocks	24

3.9.1	SyncE	24

3.9.2	Miscellaneous clocks	26

3.10	IO FPGA	26

3.11	LED Daughter Card	26

3.12	Trust 2.0	26

3.13	Fan tray connector	27

3.14	Current Measurement	27

3.15	JTAG Chain	29

3.16	Power	30

3.16.1	Voltage Margin	30

3.17	M.2 Expansion Flash	30

4	Software Considerations	32

4.1	Memory Maps	32

4.2	Programming Requirements / Suggestions	32

4.2.1	BIOS	32

4.2.2	BCM57412	32

4.2.3	BCM82752/BCM82757	32

4.2.4	FPGA Programming Requirements	33

4.3	Other Software Considerations	33

4.3.1	Cookie EPROM	33

5	User Interface Considerations/ User Experience	34

5.1	LEDs	34

6	Power Supply Requirements	35

6.1	Power Budget	35

6.1.1	Power Budget Summary	35

6.2	Main Power Supplies	35

7	Mechanical Description	36

8	Signal Integrity Considerations	37

8.1	Impedance Control	37

8.2	Signal Crosstalk	38

8.3	PCB Routing Requirements	38

8.4	PCB Stackup	38

8.5	Simulation Results	39

9	Testing Considerations	40

10	Testability Considerations	41

11	Manufacturing Considerations	42

11.1	System, Board Part Numbers	42

11.2	Programmable Part Numbers	42

11.3	Power Supply Part Numbers	43

12	Estimated Costs	44

13	Requirements Traceability Considerations	45

14	References	46

15	Standards	47

15.1	Safety Standards	47

15.2	EMC Standards	47

16	Glossary	49

17	Attachments	50

17.1	Review Action Items	50

17.1.1	Review Notes	50

Table of Figures

Figure 1. Fugazi Bezel Side View. (Not to scale.)	9

Figure 2. Fugazi Business Side View. (Not to scale.)	9

Figure 3. Fugazi High Level Block Diagram	11

Figure 4. Skylake-D Block Diagram	12

Figure 5. Skylake-D Flexlible HSIO	13

Figure 6. Fugazi UARTs	16

Figure 7. BCM87412 block diagram – from datasheet	20

Figure 8. BCM54194 Quad 1G PHY block diagram – from datasheet	21

Figure 9. BCM82757 Dual 10G PHY block diagram – from datasheet	22

Figure 10. SMI addresses for 1G Quad PHYs	22

Figure 11. SMI addresses for 10G Dual PHYs	23

Figure 12. Fugazi I2C Busses	24

Figure 13. Fugazi SyncE Clock Tree	25

Figure 14. Trust 2.0 implementation using Aikido.	27

Figure 15. Trust 2.0 implementation using Aikido – proto type only.	27

Figure 16. Fugazi 1RU System Current Sensors	27

Figure 17. Fugazi JTAG Chain	30

Figure 18. M.2 NVME/SATA/USB Block Diagram	31

Figure 19. Fugazi Motherboard 14-layer Stackup	39

Table of Tables

Table 1. Fugazi High level Hardware Feature List	7

Table 2. Fugazi Risks.	7

Table 3. Control Plane CPU Manufacturers Part Numbers.	11

Table 4. Skylake-D CPU USB Bus Mapping	14

Table 5. Skylake-D CPU SOC PCIe Port Mapping	14

Table 6. Skylake-D CPU Clock Output Mapping	14

Table 7. Supported DDR4 DIMMs.	15

Table 8. RJ-45 CON/AUX Pin Out.	17

Table 9. Micro-USB Console / AUX dongle Pin Out.	17

Table 10. USB 3.0 Type-A Pin Out.	18

Table 11. USB 3.0 Type-C Pin Out.	18

Table 12. SFP/SFP+ pin-out	23

Table 13. IDT8A35004 input clock Specifications.	25

Table 14. IDT8A35004 output clock Specifications.	25

Table 15. Miscellaneous Clocks for Fugazi	26

Table 16. Fugazi Fan Connector Pinout	27

Table 17. Fugazi Motherboard Input Current Sensing	28

Table 18. Fugazi Fan Current Sensing	29

Table 19. Supported M.2 NVME Modules	31

Table 20. M.2 Expansion Slot Pin to Module Description.	31

Table 21. Fugazi LEDs.	34

Table 22. Curie Radium High Level Power Budget	35

Table 23. Signal Crosstalk Requirements	38

Table 24. System Part Numbers.	42

Table 25. Fugazi Programmable Part Numbers.	42

Table 26. Fugazi Power Supply Part Numbers.	43

Table 27. Fugazi required EMC Standards.	47

Table 28. Review Action.	50

# Overview

The Fugazi program is a low-end ASR platform that leverages the software-based forwarding technology targeting 20Gbps performance on low-cost System on a Chip (SoC) CPUs and fills a space between the high-end ISR products and the ASR hardware-based forwarding platforms.

High-level designs features are shown in Table 1. The table is copied from the Fugazi System Functional Specification, EDCS-15396836. Refer to the SFS for any discrepancies.

Table 1. Fugazi High level Hardware Feature List

| Feature |  |
| --- | --- |
| Form Factor | 1RU |
| Data Plane CPU/Memory | Single SoC with shared CPU and memory between control and data plane |
| System CPU | Intel Skylake-D 2.5G+ 12C |
| System Memory Default | 2xDIMM slots, 1x16GB DDR4 default |
| System Memory Upgrade Options | 32GB and 64GB |
| Boot Flash | 16GB soldered down. |
| SSD | M.2 (600GB NVME, 2TB NVME, 16GB USB, or 32GB USB) |
| Front Panel Ports | 4x10G SFP+ 8x1GE SFP |
| Management GE Port | 1 x RJ45 10/100/1000Mbps |
| Console Port | 1xRJ45 and 1xUSB micro type-B |
| Aux Port | Support via micro-USB (Star dongle) without modem control signals |
| USB Port | 1xUSB 3.0 Type-A (1100mA) 1xUSB 3.0 Type-C (1.5mA) |
| Fans | Internal |
| NEBS | Yes |
| Removable Main PSU (Redundant Mode) | 2x 500W AC 2x 437W DC 2x 400W HVDC/AC |

## Risks

Most components that are used in Fugazi are used in the Curie 1RU, 2RU and Green Day projects.  The risk for Fugazi is therefore low.

Table 2. Fugazi Risks.

| Risk | Mitigation |
| --- | --- |
| Aikido implementation of Trust 2.0 | Similar to Curie and Green Day/Ramones. No mitigation. |
| SyncE | Similar implementation to Green Day/Ramones. No mitigation |
| Performance | SW based forwarding plane – new to ASR platforms. |

## Design Considerations

### NEBS requirement

NEBS will be required.  No special airflow will be necessary.

### Hardware Process Requirements

Fugazi will follow the guidelines set out in CHG Hardware Development Process, EDCS-1511434 [8].

Also, Fugazi will follow normal Cisco Derating Guidelines as specified in [9].

# High-Level Description

## Configurations

### Fugazi Bezel Side

The side where the fans are located is referred to as the bezel side – even though there is no separate bezel present on this box.  The bezel side has the 4fans (numbered left to right) and access to 2 PSU power supply bays can be found.  The On/Off switch is used to turn on and gracefully turn off the system; no separate on/off switch exists on the power supplies.  One power supply can be removed while the system is powered by another power supply; however, the AC cable must be removed before power supply removal latch can be disengaged.

The bezel side has the following interfaces:

- 2 PSU Slots

- System Power On/Off switch

- Fan exhaust

Figure 1. Fugazi Bezel Side View. (Not to scale.)

### Fugazi Business (I/O Panel) Side

The business side has access to Ethernet ports.  The following interfaces are available:

- 1 USB 3 Type A host ports

- 1 USB 3 Type C host ports

- 1 USB micro-Type B Console port

- 1 RJ45 Console port

- 1 10/100/1000 RJ-45 Management Ethernet port

- 8 1G SFP Ethernet ports (labeled “GE0/0/0-GE0/0/7”)

- 4 1G/10G SFP/SFP+ Ethernet ports (labeled “TE0/1/0-TE0/1/3”)

- M.2 Flexible NVME / USB slot

- LEDs for Ethernet and console status

- Status/Alarm LEDS and backlit Cisco Logo

Figure 2. Fugazi Business Side View. (Not to scale.)

## Performance

Refer to the Fugazi System Functional Specification, EDCS-15396836.

## Functional Blocks

See section 3.

## Theory of Operation

Refer to the Fugazi System Functional Specification, EDCS-15396836.

## Initiative, Legal, & Regulatory

N/A

# Detailed Description

## System Block Diagram

A high-level block diagram is shown in Figure 3.

Figure 3. Fugazi High Level Block Diagram

## Intel Skylake-D Control Plane CPU

### Control Plane CPU Part Numbers

Table 3 shows the Control Plane CPUs.

Table 3. Control Plane CPU Manufacturers Part Numbers.

| # Port | # Fugazi |
| --- | --- |
| Vendor | Intel |
| SKU | D-2168NT |
| MPN | FH8067304217500 QRJF |
| CPN | 15-106911-01 |
| Cores | 12 |
| Cache | 16.5MB |
| Speed | 2600MHz |
| Power | 110W |

### Skylake-D CPU

Fugazi uses a 12-core Skylake-D processor from Intel’s Xeon D-2100 product family. Figure 4 shows a high-level block diagram of Skylake-D. The main features of the processor include:

- 2.6GHz Xeon Processr

- 12 cores

- 1MB/Core Cache

- Up to 4 DDR4 channels (up to 2DIMMs / channel, up to 2133 MT/s data rate)

- Up to 4 ports 10 GbE/1GbE Integrated Ethernet (Supports KR/KX/SFI) supporting SR-IOV

- x32 PCIe Gen3 + up to 20 PCIe Gen3 lanes via Flexible IO

- 20 High-Speed Flexible Lanes configured as PCIe or SATA or USB3.0 (Up to 20 lanes of PCIe 3.0 or Up to 14 lanes of  SATA Gen 3 or Up to four ports of USB Gen 3)

- FCBGA 45 mm x 52.5 mm, Socket-less FCBGA-2518

Figure 4. Skylake-D Block Diagram

### Skylake-D DDR4

Skylake-D has 4 DDR4 controllers. The Intel documentation refers to these as 0, 1, 3, 4 and A, B, D, E; in this HFS and schematic documentation the integer notation will be used.  Fugazi will use controller 1 and 4, and they are located physically on either side of the CPU.  The selection is based on ease of routing.  Each channel will have one DIMM slot.

DDR4 channel 0/1 and 3/4 have separate voltage domains, hence two separate voltage rails are required.

Note that there are several rules that apply to the memory.  From the PDG the following is listed for mixing DIMMs:

- All DIMMs must be DDR4 DIMMs.

- x4 and x8 DIMMs can be mixed in the same channel.

- Mixing of LRDIMMs and RDIMMs is not allowed in the same channel, across different channels, and across different sockets.

- Mixing of non-3DS and 3DS LRDIMMs is not allowed in the same channel, across different channels, and across different sockets.

- Mixing of DDR4 operating frequencies is not validated within a socket or across sockets by Intel. If DIMMs with different frequencies are mixed

The I2C addressing for the DIMMs will be 0xA4 for both.  This is specified in the PDG table 6-3.

### Skylake-D Flexible HSIO

Skylake-D has 26 flexibile High Speed I/O ports that supports various controllers; 20 HSIOs are available for the user.  An overview from the CPU datasheet is shown in Figure 5.

Figure 5. Skylake-D Flexlible HSIO

Fugazi uses 2 USB3 lanes but should be configured for 4 lanes, and 16 PCIe lanes.  See 3.2.5 for USB mapping and 3.2.6 for PCIE mapping

### CPU USB Mapping

Table 4 shows the USB port mapping.

Table 4. Skylake-D CPU USB Bus Mapping

| CPU USB port | Neptune |
| --- | --- |
| USB 2.0, Port 0 | Front-Panel USB Type A, Port 0 |
| USB 2.0, Port 1 | Front-Panel USB Type C, Port 1 |
| USB 2.0, Port 2 | eMMC controller |
| USB 2.0, Port 3 | M.2 USB slot |
| USB 3.0, Port 1 | Front-Panel USB Type A, Port 0 |
| USB 3.0, Port 2 | Front-Panel USB Type C, Port 1 |
| USB 3.0, Port 5 | Not used |
| USB 3.0, Port 6 | Not used |

### CPU PCIe Mapping

Table 5 shows the PCIe port mapping and bifurcation for the Skylake-D CPU.

Table 5. Skylake-D CPU SOC PCIe Port Mapping

| PCIe Block | Lane Num, Lane Width | HSIO lane | Symbol Port | Fugazi |
| --- | --- | --- | --- | --- |
| CPU PCIe,Gen 3, PCIE1, x16 (2x4) | [3:0], x4 |  | PCIE1[3:0] | 10G MAC4 (x4, g3) |
|  | [7:4], x4 |  | PCIE1[7:4] | 10G MAC5 (x4, g3) |
|  | [11:8], x4 |  | PCIE1[11:8] |  |
|  | [15:12], x4 |  | PCIE1[15:12] | NVME (x4,g3) |
| CPU PCIe, Gen 3, PCIE2, x16 (4x4) | [3:0], x4 |  | PCIE2[3:0] | 10G MAC0 (x2, g3) |
|  | [7:4], x4 |  | PCIE2[7:4] | 10G MAC1 (x2, g3) |
|  | [11:8], x4 |  | PCIE2[11:8] | 10G MAC2 (x2, g3) |
|  | [15:12], x4 |  | PCIE2[15:12] | 10G MAC3 (x2, g3) |
| PCH PCIe, Gen 2, (4x2,2x4) | [4], x1, gen 2 | [10] | PCIE0[4] | FPGA (x1, g1) |
|  | [5], x1, gen 2 | [11] | PCIE0[5] | - |
|  | [6], x1, gen 2 | [12] | PCIE0[6] | i211 (x1, g1) |
|  | [7], x1, gen 2 | [13] | PCIE0[7] | - |
|  | [9:8], x2, gen 2 | [15:14] | PCIE0[9:8] |  |
|  | [11:10], x2, gen 2 | [17:16] | PCIE0[11:10] |  |
|  | [15:12], x2, gen 2 | [21:18] | PCIE0[15:12] |  |
|  | [20:16], x2, gen 2 | [25:22] | PCIE0[20:16] |  |

### CPU PCIe Output Clock Mapping

Table 6 shows the PCIe clock port mapping for the Skylake-D CPU. Note that there are 15 outputs in addition to a separate XDP output.  Outputs 3, 4, and 5 have been selected for the 3 CPU BLK inputs.

Table 6. Skylake-D CPU Clock Output Mapping

| PCIE clock outputs | Connected Device | Speed |
| --- | --- | --- |
| CLK_PCIE_0 | Unused | 100MHz |
| CLK_PCIE_1 | Unused | 100MHz |
| CLK_PCIE_2 | Unused | 100MHz |
| CLK_PCIE_3 | CPU BCLK0 | 100MHz |
| CLK_PCIE_4 | CPU BCLK1 | 100MHz |
| CLK_PCIE_5 | CPU BCLK2 | 100MHz |
| CLK_PCIE_6 | 10G Dual MAC0 | 100MHz |
| CLK_PCIE_7 | 10G Dual MAC1 | 100MHz |
| CLK_PCIE_8 | 10G Dual MAC2 | 100MHz |
| CLK_PCIE_9 | 10G Dual MAC3 | 100MHz |
| CLK_PCIE_10 | 10G Dual MAC4 | 100MHz |
| CLK_PCIE_11 | 10G Dual MAC5 | 100MHz |
| CLK_PCIE_12 | Unused |  |
| CLK_PCIE_13 | FPGA |  |
| CLK_PCIE_14 | i211 GE MAC |  |
| CLK_PCIE_15 | NVME | 100MHz |
| CLK_PCIE_ITP_XDP | XDP | 100MHz |

### CPU Ethernet Mapping

Skylake-D’s native 10G Ethernet ports are not used in Fugazi.

### CPU RTC

The Skylake-D PCH built in RTC and CMOS memory is used in this design. These are backed-up with a Lithium coin cell CR2032 style battery.

When the system is powered from an external input source the battery is not in use.  The expected battery shelf life is:

IVCCRTC,typ = 6µA

Capacity = 220mAh

Calculated Lifetime = 220mAh / 6µA = 36667h = ~ 4 years and 2 months

## DDR4 Memory DIMMs

The Intel CPU will have 2 DDR4 DIMM slot. Fugazi plan to support 8GB, 16GB or 32GB RDIMM DIMM.  DDR4 will run 2133MT/s. Table 7 shows the planned DDR4 CPNs.  Note that these could change during the course of the project.

Table 7. Supported DDR4 DIMMs.

| # Size | # CPN | # Description |
| --- | --- | --- |
| 8GB | 15-107068-01 | IC-DRAM-MOD,DDR4, 8GB (1Gx72),1R,1333MHz(2666MT/s),1.2V NOM, 0°C to 85°C,RDIMM,288P,31.25mm H,8K Cycles,SRGBU Only |
| 16GB | 15-107070-01 | C-DRAM-MOD,DDR4,16GB(2Gx72),2R,1333MHz(2666MT/s),1.2 V NOM,0°C to 85°C,RDIMM,288P,31.25mm H,8K Cycles, SRGBU Only |
| 32GB | 15-107071-01 | IC-DRAM-MOD,DDR4,32GB(4Gx72),2R,1333MHz(2666MT/s),1.2 V NOM,0°C to 85°C,RDIMM,288P,31.25mm H,8K Cycles, SRGBU Only |

## Bootflash

The bootflash is a soldered down eMMC device. A 16GB device will be used and configured in pSLC mode; this is similar to existing ISR4K systems – please refer to Utah, Sword, Dagger Hardware Functional Specification: EDCS-1202397.

## UARTs (Serial Ports)

Figure 6 shows UART busses on Fugazi.  The CPU has no UART blocks, hence the console and debug (“AUX”) UARTs will be implemented in the FPGA.  UART0 go to the RJ-45 console port or USB console.  UART1 will be used for debugging only by providing what software refers to as the “AUX” port.  Note that the ‘Star Dongle’ would be required to get the AUX functionality through the Star Dongle’s RJ-45 connection.

In addition, the “AUX” port can be used as a debug port for the FPGA NIOS uP code.

For details on how to configure the multiplexers, please refer to FPGA HFS.

Figure 6. Fugazi UARTs

The front panel Console comply with the Connectors and Cabling Requirements, ENG-97748 document with the exception of DTR and the use of modem signals.

### CON RS-232 Transceiver

The transceiver is the MAX3222 which has 2 transmitter blocks and 2 receiver blocks for the RS-232 port.  It requires +3.3V power supplies; +/-5.5V is generated internally from a regulated dual charge pump and only one external 0.1uF ceramic capacitor for proper operation. The MAX3222 has ESD protection up to 15KV.

For the console, the DTR signal is low (system side) when the RJ-45 connector is selected, and high when the USB console is selected.

### RJ-45 Connector Pin Out

Table 8 shows the pin out for the CON RJ-45 connector.  Only TXD and RXD will be used for the console connection and RTS and CTS are shorted together.

Table 8. RJ-45 CON/AUX Pin Out.

| Pin | Signal | Direction |
| --- | --- | --- |
| 1 | RTS | Output |
| 2 | DTR | Output |
| 3 | TXD | Output |
| 4 | GND | - |
| 5 | GND | - |
| 6 | RXD | Input |
| 7 | DSR/DCD | Input |
| 8 | CTS | Input |

### USB Console

The USB Console port will be serviced by a UART to USB device transceiver (15-11263-01).  If the USB Console Port is active (cable inserted and remote PC drivers enabled), the UART mux in the FPGA should be configured appropriately; by default the console will switch from RJ-45 to USB when the USB cable is detected.   USB Console does not support hardware-flow control.   For details on this implementation please refer to the USB Console Usage and Design Specification, EDCS-654458.

#### AUX connection through USB console port.

AUX functionality in the sense that it is needed for software debug is provided on the micro-USB connector by using the dongle that has been developed for the Star fixed platforms.  The dongle has an RS232 driver that allows an RJ-45 UART cable to connect to the dongle and the UART signals will be multiplexed directly to the host FPGA.

#### Micro-USB Console Port Pin-Out

Table 9 shows the pinout for the micro-USB connector in either USB mode or when using the dongle.

Table 9. Micro-USB Console / AUX dongle Pin Out.

| Pin | Signal | USB | Dongle |
| --- | --- | --- | --- |
| 1 | +5V VCC | 5V Power | 5V Power |
| 2 | D- | Data - | TXD output to Dongle |
| 3 | D+ | Data + | RXD input from Dongle |
| 4 | ID | USB console cable has this floating. | Dongle has ID grounded |
| 5 | GND | GND | GND |

## Front USB Ports

The system has two front panel USB ports: 1 USB 3 Type-A and 1 USB 3 Type-C.  They both support USB 2.0 and USB 3.0 at 5Gbps.

### USB 3.0 Type-A

The USB 3.0 Type-A connector supports USB 2.0 and USB 3.0 signaling.  The pin-out is shown in Table 10.  Signal naming for the differential pairs are relative to the host side.  High speed signaling is 5Gbps and re-drivers are used for signal conditioning – please see section 3.6.3.

Table 10. USB 3.0 Type-A Pin Out.

| Pin | Signal | Description |
| --- | --- | --- |
| 1 | VBUS | 5V Power |
| 2 | D- | USB 2.0 Data - |
| 3 | D+ | USB 2.0 Data + |
| 4 | GND | GND |
| 5 | SSRX- | USB 3.0 Receive |
| 6 | SSRX+ |  |
| 7 | GND | GND (high speed signals) |
| 8 | SSTX- | USB 3.0 Transmit |
| 9 | SSTX+ |  |

Power for VBUS is controlled by a TPS2561 adjustable current limiter; current limiting is set to 1.4A nominal to allow for a guaranteed minimum 1.1A, or ~5.5W power delivery.

### USB 3.0 Type-C

The USB 3.0 Type-C connector supports USB 2.0 and USB 3.0 signaling.  The pin-out is shown in Table 11.  Signal naming for the differential pairs are relative to the host side.  High speed signaling is 5Gbps and re-drivers are used for signal conditioning – please see section 3.6.3.

Table 11. USB 3.0 Type-C Pin Out.

| Pin | Signal | Description | Pin | Signal | Description |
| --- | --- | --- | --- | --- | --- |
| A1 | GND | Ground | B12 | GND | Ground |
| A2 | TXp1 | First TX differential pair | B11 | RXp1 | First RX differential pair |
| A3 | TXn1 |  | B10 | RXn1 |  |
| A4 | VBUS | Power | B9 | VBUS | Power |
| A5 | CC1 | Configuration Channel | B8 | SBU2 | Sideband Use |
| A6 | Dp1 | USB 2.0 Position 1, D+ | B7 | Dn2 | USB 2.0 Position 2, D- |
| A7 | Dn1 | USB 2.0 Position 1, D- | B6 | Dp2 | USB 2.0 Position 2, D+ |
| A8 | SBU1 | Sideband Use | B5 | CC2 | Configuration Channel |
| A9 | VBUS | Power | B4 | VBUS | Power |
| A10 | RXn2 | Second RX differential pair | B3 | TXn2 | Second TX differential pair |
| A11 | RXp2 |  | B2 | TXp2 |  |
| A12 | GND | Ground | B1 | GND | Ground |

The USB Type-C protocol requires a DFP (Downstream Facing Port) controller on the host side.  A TPS25810 integrated controller and power switch is used on the platform; the controller will monitor the CC1 and CC2 pins on the connector and utilize this information to enable VBUS power and set the polarity for the SerDes multiplexer.

Power for VBUS is set to the 1.5A mode; the TPS25810 provides nominal 1.7A and minimum 1.58A for a ~7.5W power delivery.

A Type-C connector is not keyed and can be oriented in two different ways.  A high-speed SerDes mux, HD3SS3212 equivalent, is used to steer the USB 3.0 TX/RX signals to the first or second differential pair on the connector.  The mux select and output enable is controlled by the TPS25810.

### USB High Speed Re-driver

Due to the distance from the CPU complex to the front-panel USB ports, high-speed re-drivers are used to boost the 5Gbps USB 3 signals.  The re-driver used is a PI3EQX7841 or equivalent device with I2C control. The re-driver settings will be programmed by the NIOS environmental controller and adjusted based on bringup lab measurements.

For Fugazi, the maximum settings have provided the most open eye as observed on a high-speed oscilloscope.

## Front Panel Ethernet Ports

The front panel Ethernet port connections are shown in Figure 3.

### i211 GE MAC/PHY Management Port

Fugazi has a 10/100/1000 GE RJ-45 port for management.  An Intel i211 single port MAC/PHY is used to provide the GE connectivity through a CPU PCIe port.  MAC addresses will have to be programmed into the onboard EEPROM on the i211, and diagnostics will provide tools for this.

### Dual 10G MAC, BCM57412

The BCM57412 is an integrated MAC/PHY LAN controller with support of single 25 GbE ports or dual 10 GbE ports (in Fugazi dual 10G GbE ports is used). Each port can support one of XFI, SFI, 10GBase-KR / CR / SR / LR / ER interfaces. The BCM57412’s host interface supports up to x8 PCIe Gen3 lanes.  Fugazi uses x4 PCIe Gen3 lanes for the 10G MACs used for the 10G ports and x2 PCIe Gen3 lanes for the 10G MACS used for the 1G ports.

Figure 7 shows a block diagram copied from the BCM57412 datasheet.

Figure 7. BCM87412 block diagram – from datasheet

The BCM57412 has a SPI interface to connect an external ROM which stores the required boot code, MAC address and configurations.

The management interface can be SMBus, NC-SI or PCIe. In Fugazi, the PCIe interface is used for management.

BCM57412 integrates optical module sideband interfaces to control the external SFP+ optical modules.

BCM57412 has a UART interface for debug purpose which will not be connected on Fugazi.

During the course of Fugazi bringup it has been determined that the default PCIe preset value for BCM57412 is not optimal and can cause PCIe retraining events and in the worst case downgrade from gen 3 to gen1 speed.  Testing shows that PCIe Preset 4 is the optimal setting and the configuration files must be updated to reflect this.

### Quad GE PHY BCM54194

BCM54194 uses a quad 1G PHY for each 2 SFPs on Fugazi.  It is mainly used to provide MacSec support and Cisco Cleartag.  The main features applicable to Fugazi are

- SFP Media interface support 1000BASE-X (100M SFPs supported for SGMII upstream applications)

- SGMII or QSGMII interface for System side

- Support for SyncE recovered clocks

- Energy Efficient Ethernet IEEE 802.3az support

- BCM54194 supports 256 bits MACSEC & Cleartag.

- 1.0V core voltage, 3.3V I/O voltage, MDC/MDIO can run at 1.2V/2.5V/3.3V

Figure 8 shows a simple block diagram of the BCM54194.

Figure 8. BCM54194 Quad 1G PHY block diagram – from datasheet

As shown in Figure 3, the SFP’s sideband and I2C interfaces will be controlled by the BCM57412 MAC and not the BCM54194 PHY.  One of the two recovered line side clocks will be routed to the SyncE PLL and internal selection of the line side clock is required to be configured – see section 3.9.1.

### Dual 10G PHY BCM82757

BCM82757 is a dual 10G PHY used for SFP/SFP+ on Fugazi.  It is mainly used for MacSec support and Cisco Cleartag, but also provides LRM SFP+ support.  The main features applicable to Fugazi are

- Dual ports, 10GbE XFI-to-SFI PHY

- Support SFP+ SR, LR, CR, and LRM optical interface

- Support for SyncE recovered clocks

- Supports 1000BASE-X for 1GbE operation

- Support for advanced SFP+ diagnostics, e.g. eye-mapping, SNR and BER

- Support for 256 bits MACSEC

- Supports Cisco Cleartag 3.4

- 1.0V core voltage and 1.2V/2.5V/3.3V I/O voltage

- 156.25MHz differential clock input

Figure 9 shows a simple block diagram of the BCM82757.

Figure 9. BCM82757 Dual 10G PHY block diagram – from datasheet

As shown in Figure 3, the SFP’s sideband and I2C interfaces will be controlled by the BCM57412 MAC and not the BCM82757 PHY.  The two recovered line side clocks will be routed to the SyncE PLL – see section 3.9.1.

### 1G and 10G PHY SMI Address Map

The SMI (MDIO/MDC) bus is used by the BCM57412 10G MAC as control interface to the PHYs.  Some SMI addresses are required by the host software to be unique and the addresses highlighted in Figure 10 and Figure 11 is planned for software use.  The strapping for each PHY is done in hardware.

Figure 10. SMI addresses for 1G Quad PHYs

Figure 11. SMI addresses for 10G Dual PHYs

### SFP/SFP+ Interface

Each SFP port will support a number of different SFP modules as listed in the PRD. The SFP pin-out is show in Table 12.

Table 12. SFP/SFP+ pin-out

| Bottom of SFP module |  | Top of SFP module |  |
| --- | --- | --- | --- |
| Pin | Description | Pin | Description |
| 1 | VeeT | 11 | VeeR |
| 2 | TxFault | 12 | RD- |
| 3 | TxDisable | 13 | RD+ |
| 4 | MOD-DEF(2) | 14 | VeeR |
| 5 | MOD-DEF(1) | 15 | VccR |
| 6 | MOD-DEF(0) | 16 | VccT |
| 7 | RateSelect | 17 | VeeT |
| 8 | LOS | 18 | TD+ |
| 9 | VeeR | 19 | TD- |
| 10 | VeeR | 20 | VeeT |

The SFP uses LVPECL signaling for data transmission and low speed TTL signals for status and control signals. For the LVPECL serial interface, 100 differential signaling (each signal’s impedance is 50) is used. The low speed control interface provides transmitter control, transmitter monitoring and module identification. MOD-DEF0 is grounded on the SFP and used for insertion/removal detection. MOD-DEF[2:1] are used as SDA and SCL respectively and gives access to the ID PROM on the SFP where module information is stored.

The SFP requires 3.3V supply on the Vcc,x pins and pins Vee,x shall be connected to ground. OIR is provided in the form of a 1.1A nominal rated PTC that trips at around 2.2A.

TxFault is an output from the SFP indicating an error condition in the laser, TxDisable is an input to the SFP signaling to the module to disable the transceiver, and LOS is an output from the SFP indicating loss of signal in the receiver. These three control/status signals along with MOD-DEF[2:0] will be accessible from the FPGA.

## I2C

The I2C busses for Fugazi is shown in Figure 12. More details for FPGA access are provided in Curie 1RU FPGA Hardware Functional Specification, EDCS-15093645 .

Figure 12. Fugazi I2C Busses

## Clocks

### SyncE

Synchronous network clocks are created by the 8A35004 SETS device. The IDT8A35004 received clock inputs from the 1G/10G PHYs and will output clocks to both 1G/10G PHYs and through a clock buffer to the 10G MACs.  I high level clock tree for IDT8A35004 is shown in Figure 13.  Detailed clock specifications are shown in Table 13 and Table 14.

Figure 13. Fugazi SyncE Clock Tree

Table 13. IDT8A35004 input clock Specifications.

| Input | Source Device | Frequency | Type |
| --- | --- | --- | --- |
| CLK0 p | BCM54194 Quad 1G PHY0 “REC_CLK1” | 25MHz | CMOS |
| CLK0 n | BCM54194 Quad 1G PHY1 “REC_CLK1” | 25MHz | CMOS |
| CLK1 p | BCM54194 Quad 1G PHY2 “REC_CLK1” | 25MHz | CMOS |
| CLK1 n | BCM54194 Quad 1G PHY3 “REC_CLK1” | 25MHz | CMOS |
| CLK2 p | unused |  |  |
| CLK2 n | unused |  |  |
| CLK3 p/n | BCM82757 DUAL 10G PHY 4 “CLK_RCVCLK0” | 156.25MHz | CML, AC-coupled, 50Ω internal |
| CLK4 p/n | BCM82757 DUAL 10G PHY 4 “CLK_RCVCLK1” | 156.25MHz | CML, AC-coupled, 50Ω internal |
| CLK5 p/n | BCM82757 DUAL 10G PHY 5 “CLK_RCVCLK0” | 156.25MHz | CML, AC-coupled, 50Ω internal |
| CLK6 p/n | BCM82757 DUAL 10G PHY 5 “CLK_RCVCLK1” | 156.25MHz | CML, AC-coupled, 50Ω internal |

Table 14. IDT8A35004 output clock Specifications.

| Output | Destination Device | PLL | Frequency | Type |
| --- | --- | --- | --- | --- |
| Q0_P/N | Unused (Clock buffer for BCM57412 core clock, but not installed) | DPLL0 | 50MHz | LVPECL |
| Q1_P | FPGA | DPLL0 | 25MHz | CMOS |
| Q1_N | Unused | DPLL0 | 25MHz | CMOS |
| Q2_P/N | BCM54194 Quad 1G PHY0 “REF_CLK+/-” | DPLL1 | 125MHz | LVPECL, AC-coupled |
| Q3_P/N | BCM54194 Quad 1G PHY1 “REF_CLK+/-” | DPLL1 | 125MHz | LVPECL, AC-coupled |
| Q4_P/N | BCM82757 DUAL 10G PHY 4 “REFCKP/N” | DPLL2 | 156.25MHz | 1VPP, AC-coupled |
| Q5_P/N | BCM82757 DUAL 10G PHY 5 “REFCKP/N” | DPLL2 | 156.25MHz | 1VPP, AC-coupled |
| Q6_P/N | BCM54194 Quad 1G PHY2 “REF_CLK+/-” | DPLL3 | 125MHz | LVPECL, AC-coupled |
| Q7_P/N | BCM54194 Quad 1G PHY3 “REF_CLK+/-” | DPLL3 | 125MHz | LVPECL, AC-coupled |

### Miscellaneous clocks

Table 15 lists clocks and crystals not previously defined above.

Table 15. Miscellaneous Clocks for Fugazi

| Source | Speed | Target Device | Input Clock | Interface |
| --- | --- | --- | --- | --- |
| Intel Broadwell | 24MHz | Intel Skylake-D | CLK_24M0_PMSYNC | 1.05V Single Ended |
| Intel Broadwell | 24MHz | Mictor Header | CLK_24M0_LPC_DBG | 3.3V CMOS |
| Intel Broadwell | 24MHz | FPGA | CLK_24M0_LPC_FPGA | 3.3V CMOS |
| XTAL | 25 MHz | Intel Skylake-D | XTAL_IN/OUT | XTAL |
| OSC | 25MHz | FPGA | CLK25P0M | 3.3V CMOS |
| XTAL | 24 MHz | USB2642 | XTAL1/2 | XTAL |
| XTAL | 32.7680 KHz | Intel Skylake-D | RTCX1/X2 | XTAL |

## IO FPGA

Fugazi has an IO FPGA similar to existing ISR4K platforms.  For details please refer to Curie 1RU FPGA Hardware Functional Specification, EDCS-15093645 .

## LED Daughter Card

Fugazi will have a daughter card with status LEDs and backlit logo LEDs.  The LED DC is reused from the Curie program.  See the FPLED_Board_Spec, EDCS-15985755.

For details on the LED behavior, please see section 5.1.

## Trust 2.0

To support Trust Tech 2.0 the previous ACT2 chip will not be adequate.  For initial bringup and backup, ACT2 will still be present on the HW, but it is planned to be depopulated in production.

To achieve Trust 2.0, Curie will rely on the implementation using “Aikido” as shown in Error! Reference source not found..  Aikido is an FPGA implementation using M2S005TS.  Note that the Aikido chip has the ability to reset the CP CPU autonomously of the FPGA.

Figure 14. Trust 2.0 implementation using Aikido.

For early prototypes (revision 1 and 2), Aikido was implementated by interfacing to Kingfisher in the FPGA as shown in Figure 15.  The change is transparent to software, but the FPGA and Aikido firmware images are different between the two implementations.

Figure 15. Trust 2.0 implementation using Aikido – proto type only.

## Fan tray connector

Fugazi platforms will use 4 individual 38mm fans.  The fan connector pin-out is similar to Sword and Dagger and is shown in Table 16.

Table 16. Fugazi Fan Connector Pinout

| Pin Number | Description |
| --- | --- |
| 1 | GND |
| 2 | 12V |
| 3 | TACH (input to host) |
| 4 | PWM (output from host) |

The fans are planned to have Cisco Smartfan technology for identification purposes. Please see the Cisco Smart Fan Specification, EDCS-11544582.

## Current Measurement

There are a number of current sensors in the system.  Figure 16 shows the sensors.  The power supplies have built in sensors.  The motherboard has 3 sensor locations; two for main power into the motherboard and one for power to the fans.

Figure 16. Fugazi 1RU System Current Sensors

The current sensors on the motherboard uses sense resistors and an amplifier to generate an analog voltage which is then sampled by the power sequencer.  The sense resistor needs to be designed for the appropriate power requirements and the amplifier and ADC requirements. The amplifier used is a TI INA180, Cisco part number 15-107025-01, with 50 times amplification.

Table 17 and Table 18 show the design summary for the sensors on the motherboard.

Table 17. Fugazi Motherboard Input Current Sensing

Table 18. Fugazi Fan Current Sensing

## JTAG Chain

The JTAG chain is shown at a high level in Figure 17 for Fugazi.  The JTAG chain supports:

- Normal manufacturing boundary scan

- Secure JTAG

- Access to the FPGA for programming and debugging using an Altera USB byte blaster

- Access to the Aikido FPGA for programming and debugging.

- Access to the Skylake-D CPU using an XDP header

Figure 17. Fugazi JTAG Chain

## Power

The Fugazi voltage rails and power up sequencing is described in the Fugazi Power Budget, EDCS-18301088.

### Voltage Margin

Only 3.3V can be margined. The margining is done through the I2C bus to the device; the NIOS ENV MCU control the TPS53659.

## M.2 Expansion Flash

Fugazi will support an M.2 slot for mixed use of PCIe based NVME modules and a newly defined eMMC USB card.  The M.2 expansion card can be removed externally

Figure 18 shows a block diagram of the connections.

Figure 18. M.2 NVME/SATA/USB Block Diagram

The M.2 NVME slot has connects to the CPU with 2 lanes PCIe gen3.  The planned supported parts for the NVME is shown in Table 19.

Table 19. Supported M.2 NVME Modules

| # Size | # CPN | # Description | # Vendor MPN1) |
| --- | --- | --- | --- |
| 600GB | 16-101462-01 | IC-FEPROM-M,SSD,PCIe,1000GB,3.3V,M.2,0 to 55 C,DWPD<1,SRGBU only | SSDPELKX010T801 |

1) Please use Agile for latest MPNs and qualified parts

A new USB based module that can support lower densities and be removed by the user will also be supported.  The USB module will be USB2.0 based and is expected to be 32GB maximum initially.  The USB module will use the controller (USB2642) and eMMC modules that are also used on the Curie 1RU motherboard for bootflash.

To determine the type of card that is installed, the sideband signals (existing or new) in Table 20 provided:

Table 20. M.2 Expansion Slot Pin to Module Description.

| M2_PRESENT_L (pin 75 - NEW) | USB_PRESENT_L (pin 28 – NEW) | PEDET (pin 69) | Module |
| --- | --- | --- | --- |
| 1 | X | X | Module not present |
| 0 | 1 | 0 | M.2 SATA |
| 0 | 1 | 1 | M.2 NVME |
| 0 | 0 | 0 | M.2 USB |
| 0 | 0 | 1 | M.2 with EEPROM for type |

# Software Considerations

This section describes requirements that is place on software by the hardware.

## Memory Maps

For details on the FPGA register definition, please refer to reference [1].

## Programming Requirements / Suggestions

### BIOS

BIOS needs to ensure the following at a minimum:

- SPI clock speed must be set to 20MHz when reading the BIOS content.

- Fast read must be disabled when reading the BIOS content

- Dual read from SPI is enabled

- SPI devices in that must appear in the VSCC table:

  - Macronix MX25L25635FMI-10G: 		0xC22019 	VSCC: 0x20452045

  - Micron   MT25QL256ABA8ESF-0SIT: 	0x20BA19 	VSCC: 0x20052005

- Ensure that PCIe bifurcation is set as per Table 5.

- Enable clock outputs as per Table 6

- Disable ASPM for PCH PCIe downstream devices

- May need to add routing rules for PCH PCIe INT/PIRQx.

- FPGA memory space is reserved at FED40000-FED4FFFF (TPM memory space)

- Enable SMLink0 read of CPU temperature

- Enable SMLink1 read of PCH temperature

BIOS upgrade and PROM selection is similar to existing ISR4K products.  Please see [1] for details.

### BCM57412

BCM57412 requires an external NVRAM component to store various device firmware and configuration options. NVRAM is managed by the Hardware Resource Manager (HWRM) firmware executing in the embedded processor.

- The SPI interface initially read the NVRAM’s Manufacturing and Device ID at 18.75 MHz frequency. The normal operating of SCLK frequency is 37.5 MHz.

- The NVRAM must be programmed with boot code and contain a valid set of default configuration (can be got from software kit) to ensure the chip be enumerated correctly in an OS environment.

### BCM82752/BCM82757

Based on compliance mask testing, the following changes are required changes from the default values:

- Write to register 0x4800D0A5 (TX_CTRL_5) = 0x2000. (Bit 14=1: enables pre path)

- Write to register 0x4800D110 (TX_FIR Control1 Register) = 0x00E0. (bit 9:5=0111: TX FIR post override value)

- Write to register 0x4800D111 (TX FIR Control2 Register) = 0x8028. (bit 15 = txfir_override_en)

### FPGA Programming Requirements

For details on the FPGA register definition, please refer to reference [1].

## Other Software Considerations

### Cookie EPROM

The system cookie information is documented in 61-104107-XX for the Fugazi Motherboard.

# User Interface Considerations/ User Experience

## LEDs

LEDs and LED usage is listed in Table 21.

Table 21. Fugazi LEDs.

| LED | Color | Description |
| --- | --- | --- |
| LOGO | Blue | Cisco Logo LED Off: The system is powered off Blue: The system is powered on |
| PSU | Green/ Amber | Power Supply Status Off: The system is powered off Yellow: A Power Supply in the system is not functioning correctly Green: All installed PSUs are operating correctly |
| STATUS | Green/ Amber/ Red | System Status Red: The system is booting Blinking Red: The system has failed a hardware integrity error Yellow: Rommon has completed booting and system at Rommon prompt or booting platform software. Green: Normal System Operation |
| MINOR ALARM | Amber | Minor Alarm Off: No Minor Alarm Amber: Minor Alarm Asserted |
| MAJOR ALARM | Red | Major Alarm Off: No Major Alarm Red: Major Alarm Asserted |
| CRITICAL ALARM | Red | Critical Alarm Off: No Critical Alarm Red: Critical Alarm Asserted Note that this LED is on at power on and turns off when IOS has booted. |
| LINK | Green | RJ-45 Management Ethernet Link LED Off: No link Green: Ethernet cable present and link established with other side |
| SPEED | Green | RJ-45 Management Ethernet Speed LED Off: No Link Blinking Green: blink frequency indicates port speed: 1 blink – 10Mbps link speed 2 blinks – 100Mbps link speed 3 blinks – 1000Mbps link speed |
| USB CON | Green | USB Console Active Green indicates that the active console port is USB. When this is “On” the SER CON LED will be “Off”. |
| SER CON | Green | Serial Console Active Green indicates that the active console port is RJ-45. When this is “On” the USB CON LED will be “Off”. |
| SFP | Green/ Yellow | SFP port 0/1 Link LED Off: Not present or not configured Yellow: Loss of Signal Green: Link established |

# Power Supply Requirements

## Power Budget

The detailed power budget can be found in the Fugazi Power Budget, EDCS-18301088.

### Power Budget Summary

Table 22 shows a derived high level power budget for Fugaz. The total power of a fully loaded system comes out to around 230W.  A 450W PSU will be used for Fugazi as the basic PSU.  It is expected that the MB and all modules will not run at maximum power draw.

Table 22. Curie Radium High Level Power Budget

| Platform | Qty | Power | Total Power |
| --- | --- | --- | --- |
| Fan | 4 | 10W | 40W |
| SFP | 8 | 1W | 8W |
| SFP+ | 4 | 1.5W | 6W |
| M.2 SSD | 1 | 6.5W | 6.5W |
| USB-A | 1 | 4.5W | 4.5W |
| USB-C | 1 | 7.5W | 7.5W |
| Motherboard | 1 | 157 W | 157W |
| Total |  |  | 229.5W |

## Main Power Supplies

Fugazi will use the same power supply form factor as was used on Nightster/Juno.  Please refer to section 11.3 for detailed information.

# Mechanical Description

TBD

# Signal Integrity Considerations

## Impedance Control

The below impedance control requirements apply to Fugazi Motherboard:

- Single ended trace width for external (Top/Bottom) layers = 5.5 mils. Target characteristic impedance = 55 ohms +/- 10% for single ended trace in all external layers

- Single ended trace width for external (Top/Bottom) layers = 6.5 mils. Target characteristic impedance = 50 ohms +/- 10% for single ended trace in all external layers

- Single ended trace width for internal layers = 4 mils. Target characteristic impedance = 55 ohms +/- 10% for single ended trace in all internal layers

- Single ended trace width for internal layers = 5.1 mils. Target characteristic impedance = 50ohms +/- 10% for single ended trace in all internal layers

- Differential trace width = 4.5mils and spacing = 5 mils (pitch = 9.5 mils) for external (Top/Bottom) layers. Target characteristic impedance = 100 ohms +/- 10% for differential trace in all external layers

- Differential trace width = 4.5mils and spacing = 7.5 mils (pitch = 12 mils) for internal layers. Target characteristic impedance = 100 ohms +/- 10% for differential trace in all internal layers

- Differential trace width = 7.0 mils and spacing = 6 mils (pitch = 13 mils) for external (Top/Bottom) layers. Target characteristic impedance = 85 ohms +/- 10% for differential trace in all external layers

- Differential trace width = 5.5 mils and spacing = 7.5 mils (pitch = 13 mils) for internal layers. Target characteristic impedance = 85 ohms +/- 10% for differential trace in all internal layers

- Differential trace width = 6.0 mils and spacing = 5 mils (pitch = 11 mils) for external (Top/Bottom) layers. Target characteristic impedance = 90 ohms +/- 10% for differential trace in all external layers

- Differential trace width = 4.7 mils and spacing = 5.3 mils (pitch = 10 mils) for internal layers. Target characteristic impedance = 90 ohms +/- 10% for differential trace in all internal layers

- Single ended trace width in the package BGA field is 3.5 mils with target impedance = 60 ohm +/- 10% in all internal layers.

- Single ended trace width for external (Top/Bottom) layers = 10 mils. Target characteristic impedance = 40 ohms +/- 10% for single ended trace in all external layers

- Single ended trace width for internal layers = 7.125 mils. Target characteristic impedance = 40 ohms +/- 10% for single ended trace in all internal layers

- Differential trace width = 8.1 mils and spacing = 4.9 mils (pitch = 13 mils) for external (Top/Bottom) layers. Target characteristic impedance = 80 ohms +/- 10% for differential trace in all external layers

- Differential trace width = 6.5 mils and spacing = 6.5 mils (pitch = 13 mils) for internal layers. Target characteristic impedance = 80 ohms +/- 10% for differential trace in all internal layers

## Signal Crosstalk

Table 23 shows the signal crosstalk requirements for the Fugazi platforms.

Table 23. Signal Crosstalk Requirements

| Type | Signal Speed | Minimum Crosstalk | Maximum Crosstalk |
| --- | --- | --- | --- |
| Wide bus (DDR4) | 2400 MHz | 20mV | 40mV |
| Serdes | 10G | 10mV | 20mV |
| Serdes | 8G | 15mV | 25mV |
| Serdes | 5G | 15mV | 30mV |
| Serdes | 2.5G | 20mV | 40mV |
| Clock | <500Mhz | 30mV | 50mV |

## PCB Routing Requirements

Routing requirements are specified in the schematic addprop files.

## PCB Stackup

Target PCB stack-up are shown in Figure 19.

Figure 19. Fugazi Motherboard 14-layer Stackup

## Simulation Results

Signal Integrity simulation results can be found here:

\\rtp5-netapp-ns\local\si-space10\projects\ISR\Fugazi

# Testing Considerations

The system will support the following margining for Environmental Development Validation Testing:

- Voltage high and low margining for 3.3V

# Testability Considerations

Specify any required diagnostic capabilities and special testbed requirements.  Reference any requirements that are untestable or difficult to test, with a description of how this risk factor is being mitigated, e.g., a schedule and responsibility for making all requirements testable.

<<ISO requirement>>

<Body>

# Manufacturing Considerations

## System, Board Part Numbers

Fugazi consists of the motherboard board and the LED daughtercard.  Cisco Part Numbers are shown in Table 24.  Note that the LED DC CPNs are shared with the Green Day and Ramones programs.  For more information on the LED daughter card please see section 3.11.

Table 24. System Part Numbers.

| Field | Fugazi | LED DC |
| --- | --- | --- |
| TAN |  |  |
| Elmech |  |  |
| 73- | 73-19748-xx | 73-101941-xx |
| 28- | 28-14318-xx |  |
| 60- | 60-103505-xx |  |
| 61- | 61-104107-xx |  |
| 92- | 92-103563-xx |  |
| PID | C8500L-8S4X | N/A |
| CID | 0x10E0 | N/A |

## Programmable Part Numbers

Table 25. Fugazi Programmable Part Numbers.

| RefDes | Raw CPN | 17- CPN | MPN | Description | Programming | Source Location |
| --- | --- | --- | --- | --- | --- | --- |
| U2_PSQ | 15-104664 | 17-15616 | CYSHM35925I-M068LTI | Power Sequencer | Pre-Programmed | SVN: mcu |
| U1_0V9C | 15-105513 | 17-15617 | TPS53622A | DC/DC | Pre-Programmed | SVN: power_ti |
| U1_1V2C | 15-105513 | 17-15618 | TPS53622A | DC/DC | Pre-Programmed | SVN: power_ti |
| U1_1V0C | 15-105513 | 17-15619 | TPS53622A | DC/DC | Pre-Programmed | SVN: power_ti |
| U1_1V8C | 15-105255 | 17-15620 | TPS53659 | DC/DC | Pre-Programmed | SVN: power_ti |
| U1_3V3 | 15-105255 | 17-15622 | TPS53659 | DC/DC | Pre-Programmed | SVN: power_ti |
| U6_IO | 16-4167 | 17-15388 (17-15650) | N25Q064 | FPGA | Pre-Programmed | SVN: fpga |
| U2_AK | 16-4221 | 17- 15651 | N25Q128 | Aikido, EEPROM | Pre-Programmed | SVN: |
| U1_AK | 16-100726 | 17-15500 | M2S005S | Aikido, FPGA | Pre-Programmed | SVN: |
| U1_UCN | 15-11263 | 17-8994 | CY7C64215 | USB Console | Pre-Programmed | (leveraged) |
| U8B_CSPI U7B_CSPI | 16-100597 | 17-15623 | MT25QL256 | Rommon | Pre-Programmed | (From SW team.) |
| U3_EMMC | 16-3366 |  | AT25DF041 | eMMC Controller | Diagnostics | (leveraged) |
| U2_TG0/ U2_TG1/ U2_TG2/ U2_TG3 | 16-4167-04 | 17-15877 | MX25L64 | 10G MAC | Preprogram | SVN: |
| U2_TG4/ U2_TG5 | 16-4167-04 | 17-15878 | MX25L64 | 10G MAC | Preprogram | SVN: |
| U2_NK | 16-3541 | 17-15621 | ATC24M01 | SyncE | Preprogram | SVN: |
| n/a | n/a |  | n/a | NIOS | Packaged in FPGA | SVN |

## Power Supply Part Numbers

Table 26. Fugazi Power Supply Part Numbers.

|  | 450W PSU AC | 450W DC | 450W HVDC |
| --- | --- | --- | --- |
| 341 | 341-101130-01 | 341-101131-01 | 341-101156-01 |
| 61- |  |  |  |
| 95- | 95-113186-01 | 95-113187-01 | 95-113368-01 |
| PID | PWR-CH1-400WAC | PWR-CH1-400WDC | PWR-CC1-400WHV |
| CID | 0x10F0 | 0x10F1 | 0x01F9 |

# Estimated Costs

Cost data is owned by Manufacturing

# Requirements Traceability Considerations

<<TL 9000: ADDITIONAL REQUIREMENT FOR TL 9000 COMPLIANCE>>

Requirements traceability is required for TL 9000 registration. Traceability may be performed manually, or with assistance of automated software tools.

If the requirements in the SFS are not of sufficient detail to support traceability to test cases, you may need to trace the product requirements to the more detailed functional requirements specified in this HW functional specification. In this situation, provide unique identifiers for all functional requirements and trace product requirements to functional requirements consistent with your organization’s strategy for requirements traceability. If the project has a separate traceability matrix provide a link to that document.

Refer to the Common Requirements Traceability Handbook EDCS-400506 for a thorough description of traceability and some examples of a manual implementation.

# References

Cisco Documents

- Fugazi System Functional Specification, EDCS-15396836.

- Fugazi Power Budget, EDCS-18301088

- Curie 1RU FPGA Hardware Functional Specification, EDCS-15093645

- Utah, Sword, Dagger Hardware Functional Specification: EDCS-1202397

- FPLED_Board_Spec, EDCS-15985755

- Fugazi_mother_board_Design_Review_Checklist, EDCS-18342138

- Cisco_Fugazi_Executive_Summary_Action_Tracker_R0629-477.SM01_v2.0, EDCS-15974828 (Schematic modeling results from rev1)

- CHG Hardware Development Process, EDCS-1511434

- Cisco Derating Guidelines, EDCS-517316

- Vendor Documents

# Standards

## Safety Standards

For details on the Safety test plan and standards, please refer to XXX

## EMC Standards

Table 27. Fugazi required EMC Standards.

| Category | Standard/Specification | Level/Class | Notes |
| --- | --- | --- | --- |
| EMC (Emissions) | FCC 47CFR15 AS/NZS CISPR 22 CISPR 22 EN55032 ICES-003 VCCI KN 32 CNS-13438 | Class A | 6dB margin |
|  | EN61000-3-2 Power Line Harmonics (EMI-1) EN61000-3-3 Voltage Changes, Fluctuations, and Flicker (EMI-1) EN61000-3-11 Voltage Fluctuations and Flicker (EMI-2) EN61000-3-12 Power Line Harmonics (EMI-3) |  |  |
| EMC (Immunity) | IEC/EN61000-4-2 Electrostatic Discharge Immunity | 8kV contact, 15kV air | Criteria A (EMC-1) |
|  | IEC/EN61000-4-3 Radiated Immunity | 10V/m | Criteria A |
|  | IEC/EN61000-4-4 EFT-B Immunity (AC Power Leads) | 2kV | Criteria B |
|  |  | 1kV | Criteria A |
|  | IEC/EN61000-4-4 EFT-B Immunity (DC Power Leads) | 2kV | Criteria B |
|  |  | 1kV | Criteria A |
|  | IEC/EN61000-4-4 EFT-B Immunity (Signal Leads) | 1kV (indoor) | Criteria B |
|  |  | 0.5kV (Indoor) | Criteria A |
|  | IEC/EN61000-4-5 Surge AC Port | 4kV L-GND, 2kV L-L | Criteria B |
|  | IEC/EN61000-4-5 Surge DC Port | 1kV L-GND, 1kV L-L | Criteria B |
|  | IEC/EN61000-4-5 Surge Signal Port | 1kV L-GND | Criteria B |
|  | IEC/EN61000-4-6 Immunity to Conducted Disturbances | 10Vrms | Criteria A |
|  | IEC/EN61000-4-8 Power Frequency Magnetic Field Immunity | 30A/m | Criteria A |
|  | IEC/EN61000-4-11 Voltage Dips, Short Interruptions, and Voltage Variations | Various (EMC-2) | Various (EMC-2) |
|  | KN 35 (EMI-2) |  |  |
| EMC (ETSI/EN) | EN 300 386 Telecommunications Network Equipment (EMC) (EMC-3) EN55022 Information Technology Equipment (Emissions) EN55024/CISPR 24 Information Technology Equipment (Immunity) EN50082-1/EN61000-6-1 Generic Immunity Standard (EMC-4) | OTC Class A |  |

EMI-1 Applicable to LV AC equipment with rated input current up to and including 16 A per phase.

EMI-2 Applicable to equipment with rated input current <= 75A and subject to conditional connection

EMI-3 Applicable to equipment with rated input current > 16A and <= 75A per phase.

EMC-1 Criteria A is required.  In the EMC formal report, formal ESD testing to Criteria B is permitted.   Verification to Criteria A shall be performed during de-risk or formal start.

EMC-2 Requirements for Voltage DIPs and Short Interruptions as specified in EN55024, CISPR 24, EN300386, and EN61000-6-1.

EMC-3 Location other than Telecommunications center (OTC).

EMC-4 Generic Immunity for residential, commercial, and light industrial environments.

# Glossary

TBD

# Attachments

## Review Action Items

### Review Notes

Table 28. Review Action.

| Item | Description | Resolution |
| --- | --- | --- |

End of Document